LDR-BASED HOME SECURITY LIGHTING SYSTEM

# Aim

To design and construct a light-sensing home security system using a Light Dependent Resistor (LDR), which automatically switches on an LED (representing a security/porch light) when the surrounding light level drops, simulating an automatic security light that activates after dark.

# Components Required

| S.No | Component | Qty | Specification |
| --- | --- | --- | --- |
| 1 | LDR (Light Dependent Resistor) | 1 | Standard 5mm LDR |
| 2 | Sensor / driver module | 1 | Onboard comparator with threshold potentiometer |
| 3 | LED (indicator/security light) | 1 | Green, 5mm |
| 4 | Breadboard | 1 | Standard solderless breadboard |
| 5 | Battery | 1 | 9V / battery pack |
| 6 | Jumper wires | Several | Male-male |
| 7 | Model house (demo enclosure) | 1 | Cardboard and chart paper |

# Procedure

- Place the breadboard on a flat surface and mount the sensor/driver module on it.

- Connect the LDR to the input terminals of the sensor module.

- Connect the LED to the output terminal of the sensor module, observing correct polarity (anode to positive).

- Connect the battery's positive and negative terminals to the power rails of the breadboard to supply the circuit.

- Adjust the onboard potentiometer on the sensor module to set the light-threshold sensitivity.

- Mount the assembled circuit inside the model house, with the LDR positioned to sense the interior/ambient light and the LED positioned as the security light.

- Test the circuit by exposing the LDR to bright light and then covering it or dimming the surroundings, and note the LED's response.

# Working

The LDR is a light-sensitive resistor whose resistance decreases as light intensity increases and rises sharply in darkness. It forms part of a voltage-divider network on the sensor module, whose midpoint voltage is compared against a preset threshold by an onboard comparator. In bright conditions, the LDR's resistance is low, holding the comparator output — and hence the LED — OFF. When the light level falls, the LDR's resistance rises, the divider voltage crosses the threshold, and the comparator switches its output, turning the LED ON. This automatic ON/OFF behaviour is used to simulate a security light that activates at dusk or when an intruder blocks the ambient light source, making an unoccupied home appear active and deterring intrusion.

# Observation

| S.No | Ambient Condition | LDR Resistance | LED Status |
| --- | --- | --- | --- |
| 1 | Bright light (daytime) | Low | OFF |
| 2 | Dim light (dusk) | Medium | OFF / borderline |
| 3 | Darkness (night / covered) | High | ON |

# Result / Image

The LDR-based home security circuit was successfully constructed and tested. The LED turned ON when the LDR was subjected to low light/darkness and remained OFF under bright light, confirming correct automatic operation of the light-sensing security system.

Circuit diagram :

Fig: Working prototype of the LDR-based home security lighting system